Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#04)

Use of Venus Simulator for Register Level Execution of RISC-V Arithmetic and Memory Instructions

Student Name: Huzaifa Kashif                Roll No: 24I-6501                  Section: CE-A

Date performed: 9/13/2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

Here we had to do addition, subtraction, multiplication and exponent of 2. We used the keyword for instructions so add, sub, mult. I used s0 and s1 to store A and B. and s2-s5 to store the answers of the instructions.

Code:

addi s0,x0,5 # A addi s1,x0,4 # B add s2,s0,s1 # C = A+B sub s3,s0,s1 # D = A-B mul s4,s0,s1 # E = A*B mul s5,s0,s0 # F= A*A = A^2

Output:

Step By Step Output: A=5 – > addi s0,x0,5 Adding 5 into x8 (s0) register which first saved register. B = 4 —> addi s1,x0,4 i) C= A+B —> add s2,s0,s1 ii) D= A-B —> sub s3,s0,s1 # D = A-B iii) E = A*B —> mul s4,s0,s1 iv) F = A^2 —> mul s5,s0,s0 v) Instruction; function7; rs2; rs1; function3; rd; opcode. add s2,s0,s1; 0; 9; 8; 0; 18; 51. sub s3,s0,s1; 32; 9; 8; 0; 19; 51. mul s4,s0,s1; 1; 9; 8; 0; 20; 51. mul s5,s0,s0; 1; 8; 8; 0; 21; 51.

Task 2:

Description:

Here i defined the formula. I set a as s1, c as s2, d as s3, e as s4, f as s5 and finally m (result) as s0. To test the formula i intialized each register with a number. Using x0 and addi, i was able to set those constants.

Code:

#i will define registers here: # a = s1,c=s2,d=s3,e=s4,f=s5 --- for m = s0 # to test if it work i will initalize each addi s1, x0, 10 addi s2, x0, 3 addi s3, x0, 2 addi s4, x0, 4 addi s5, x0, 5 sub s0, s1,s2 #a-c add s6, s3, s4 #d+e mul s6, s6,s5 # (d+e)*f add s0,s0,s6 # (a-c)+(d+e)*f

Output:

Initalizing all: Now m = (a-c)+(d+e)*f -> a-c -> d+e -> (d+e)*f -> (a-c)+(d+e)*f Since (10-3)+(2+4)*5 = 37. The hex 0x00000025 to decimal is 37.

Task 3:

Description:

Here i used li to set the registers with the specified hex address. Then i used the AND, OR, XOR keywords and compared the addresses with each other.

Code:

li s0, 0x00000fff #first number s0 = 0xfff li s1,0x00000f0f #second number s1= 0xf0f # reg s2 = AND, s3=OR, s4 = XOR and s2,s0,s1 or s3,s0,s1 xor s4,s0,s1

Output:

Storing 0xfff: Storing 0xf0f Then applying operators: 0xfff AND 0xf0f: 0xfff OR 0xf0f: 0xfff XOR 0xf0f:

Task 4:

Description:

Here the problem was that you could not set numbers as the methods. So i set and renamed the for loop method called ‘1:’ as ‘loop’ and the 1: numbered method as ‘end’. I also used .word to set an array of 3 numbers.

Code:

.data array: .word 10, 15, 20 .text main: la a0, array li a1, 3 jal ra, arraysum j done arraysum: # a0 = int a[] # a1 = int size # t0 = ret # t1 = i li t0, 0 # ret = 0 li t1, 0 # i = 0 loop: # For loop bge t1, a1,end # if i >= size, break slli t2, t1, 2 # Multiply i by 4 (1 << 2 = 4) add t2, a0, t2 # Update memory address lw t2, 0(t2) # Dereference address to get integer add t0, t0, t2 # Add integer value to ret addi t1, t1, 1 # Increment the iterator j loop # Jump back to start of loop (1 backwards) end: mv a0, t0 # Move t0 (ret) into a0 ret # Return via return address register done: mv a1 , a0 li a0,1 ecall

Output:

Task 5:

Description:

Here is used the .word keyword to set an array of numbers called table content. Then from s0- s9 i kept loading values from the array using the lw. To access a specific element in the array you had to set a specific offset. Since each element takes 4 bits. For tablecontent[0] you would give an offset of 0, tablecontent[1] you would give an offset of 4, tablecontent[2] you would give an offset of 8 ,and so on.

Code:

.data tablecontent: .word 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 .text main: la t0, tablecontent # loading each with a 4 bit offset since table is an array into different registers #ex. to access 6 we need to give it an offset of 8 lw s0, 0(t0) lw s1, 4(t0) lw s2, 8(t0) lw s3, 12(t0) lw s4, 16(t0) lw s5, 20(t0) lw s6, 24(t0) lw s7, 28(t0) lw s8, 32(t0) lw s9, 36(t0)

Output: