Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#03)

Apply Behavioral-level Verilog to Design and

Simulate Combinational Modules

Student Name: Huzaifa Kashif      Roll No: 24I-6501        Section: CE-A

Date performed: 6th September, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

Here I utilized the same concept for creating a full adder as I did in the last lab. Except here, I had a control variable which would be XORed with each B bit and would give the complement, thus acting like subtraction. Then each of the 4 bits is added.

Code:

module ha1(sum,carry,a,b); input a; input b; output sum; output carry; assign sum = a^b; assign carry = a & b; endmodule module ha2(sum,carry,a,b); input a; input b; output sum; output carry; assign sum = a^b; assign carry = a & b; endmodule module fa(SUM,COUT,a,b,cin); input a; input b; input cin; output SUM; output COUT; wire s1; wire c1; wire c2; ha1 half1(s1,c1,a,b); ha2 half2(SUM,c2,s1,cin); assign COUT = c1+c2; endmodule module addsub4(S,COUT,A,B,control); input [3:0] A; input [3:0] B; input control; output [3:0] S; output COUT; wire [3:0] BX; wire c1; wire c2; wire c3; assign BX[0] = B[0] ^ control; assign BX[1] = B[1] ^ control; assign BX[2] = B[2] ^ control; assign BX[3] = B[3] ^ control; fa FA0(S[0],c1,A[0],BX[0],control); fa FA1(S[1],c2,A[1],BX[1],c1); fa FA2(S[2],c3,A[2],BX[2],c2); fa FA3(S[3],COUT,A[3],BX[3],c3); endmodule module testbench(); reg [3:0] A; reg [3:0] B; reg control; wire [3:0] S; wire COUT; addsub4 uut(S,COUT,A,B,control); initial begin control = 0; A = 4'b0011; B = 4'b0010; #50; A = 4'b0101; B = 4'b0011; #50; A = 4'b0111; B = 4'b0001; #50; control = 1; A = 4'b0101; B = 4'b0011; #50; A = 4'b1001; B = 4'b0010; #50; A = 4'b0111; B = 4'b0100; #50; end endmodule

Output:

Task 2:

Description:

Here I again utilized the multiplexer logic used in the last lab. Here, however, I added an andAB and orAB that would ensure that it would be a 0 when A & B and 1 when A ¦¦ B.

Code:

module multiplex(Y,A,B,Select); input A,B,Select; output Y; wire notSelect; wire w0,w1; wire andAB; wire orAB; not(notSelect,Select); and(andAB,A,B); or(orAB,A,B); and(w0,andAB,notSelect); and(w1,orAB,Select); or(Y,w0,w1); endmodule module testMult(); reg A,B,Select; wire Y; multiplex uut(Y,A,B,Select); initial begin Select = 0; A = 0; B = 0; #50 Select = 0; A = 0; B = 1; #50 Select = 0; A = 1; B = 0; #50 Select = 0; A = 1; B = 1; #50 Select = 1; A = 0; B = 0; #50 Select = 1; A = 0; B = 1; #50 Select = 1; A = 1; B = 0; #50 Select = 1; A = 1; B = 1; end endmodule

Output:

Task 3:

Description:

Here I initialized the input to be 2-bit numbers. Here I utilized the always block. This makes sure that when the inputs change, the always block always runs. The always block contains the if-else statements that assign the right bit values. So when the bit is larger, greater is 1; if equal the equal is 1; and if the bit is less, less is 1.

Code:

module comparator(A,B,equal,greater,less); input [1:0] A; input [1:0] B; output equal; output greater; output less; reg equal; reg greater; reg less; always @(*) begin if (A == B) begin equal = 1; greater = 0; less = 0; end else if (A > B) begin equal = 0; greater = 1; less = 0; end else begin equal = 0; greater = 0; less = 1; end end endmodule module testComp(); reg [1:0] A; reg [1:0] B; wire equal; wire greater; wire less; comparator uut(A,B,equal,greater,less); initial begin A=2'b00; B=2'b00; #50; A=2'b00; B=2'b01; #50; A=2'b00; B=2'b10; #50; A=2'b00; B=2'b11; #50; A=2'b01; B=2'b00; #50; A=2'b01; B=2'b01; #50; A=2'b01; B=2'b10; #50; A=2'b01; B=2'b11; #50; A=2'b10; B=2'b00; #50; A=2'b10; B=2'b01; #50; A=2'b10; B=2'b10; #50; A=2'b10; B=2'b11; #50; A=2'b11; B=2'b00; #50; A=2'b11; B=2'b01; #50; A=2'b11; B=2'b10; #50; A=2'b11; B=2'b11; #50; end endmodule

Output: